|  | - Standard Analogue phone or SIP phone - Aluminum alloy die-casting body - Vandal resistant handset and cord - Weather proof protection to IP67 - Full keypad, with LED lamp - Wall mounting, simple installation - Inductive coupler fitted as standard for hard of hearing - Phone line powered/AC/DC powered - External power supply or PoE (SIP) - RJ45 Port for SIP connection - With LED lamp fixed on the top - CE,FCC,Rosh compliant - CE,FCC,RoHS, ISO,IP65/IP67 compliant |
| --- | --- |
| The JR100 series telephones are fully contained within a corrosion resistant cast aluminium weatherproof case with a door providing complete protection against dust and moisture ingress, resulting in a highly reliable product with a long MTBF. The unique robust handset is manufactured from moulded Bulk Molding Compound and designed specifically to withstand arduous use in all environments, armoured cord to provide additional resistance to vandalism or heavy industrial use. The JR100 series Weatherproof Telephones are designed for use by the railways, power stations, the armed services and heavy industry which require very reliable telephony under adverse conditions. Standard Analogue As a standard analogue telephone, this unit provides basic telephone functions and can be connected to any standard two-wire telephone line, including analogue PABX lines and PSTN. | SIP-VoIP TECHNICAL SPECIFICATIONS Adapter Input: 100-240VAC 50～60Hz Output:5V/1A Port: WAN: 10/100Base- T RJ-45 for LAN, Auto MDIX LAN: 10/100Base- T RJ-45 for PC, Auto MDIX Power Consumption: Idle:1.5W/ Active:1.8W Voice Features - Support 2 lines SIP, SIP 2.0 (RFC3261) - Codec: G.711A/u, G.7231 high/low, G.729, G.722,G.726 - Echo cancellation: Support G.168 and hand-free can support 96ms - Supports full duplex. - SIP support SIP domain,SIP authentication(none, basic,MD5),DNS name of server, peer to peer - SIP support 2 servers, user can through each server to calling in and out Network Features - WAN/LAN: support Bridge mode - Support PPPoE for xDSL - support VLAN - Support Stun penetration - Support DHCP get IP on WAN port. |

| TECHNICAL SPECIFICATION Casing Material Die cast aluminium, epoxy powder coated. Colors Gray, other colors to special order Handset Material Cycoloy with stainless steel spiral cord Keypad Weather resistant full Keypad Temperature Operating: -30°C to +65°C Storing: -40°C to +75°C Weather Resistance IP67 Ringing Tone Shrill warble tone 85 dBA @ 1 metre typical Hook switch Electronic / magnetic with no visible moving parts M.T.B.F. Calculated to have an M.T.B.F. in excess of 50,000 hours using Weight Up to 7 kg depending on option ANALOGUE Dialing MF (tone) Power supply Drawn from telephone line Lightning / Transients Protection to ITU-T k.21 enhanced levels Programming Can be programmed as an auto-dial phone by build-in keypad or remote programming | DIMENSIONS: (Measured in mm) APPROVALS This mark indicates compliance with the: Radio & Telecommunications Terminal Equipment Directive 2004/108/EC This mark indicates compliance with the: FCC Part 15 Subpart B Class B PACKAGE unit size: 339 *146 *126(mm) Carton size:200*300*440( mm ) Packaging materials: pearl cotton, carton Gross weight: 7KG Net weight: 6KG |
| --- | --- |